Structured Training Program: Wi-Fi

Module 1: Introduction and History of Wi-Fi

1. In which OSI layer the Wi-Fi standards/protocol fits?

The Wi-Fi and it’s protocol standards fits in the OSI layer 1 – Physcial Layer and OSI layer 2 – Datalink Layer.

Reasons for Layer 1:

- Signal Transmission: Wi-Fi operates using radio waves (2.4 GHz, 5 GHz, 6 GHz bands) to transmit raw bits between devices.

- Physical Specifications: 802.11 standards define the physical specifications, including modulation techniques (like OFDM, DSSS) and transceiver control.

- Hardware Interface: It manages the network interface card (NIC) and antenna interaction to put data onto the wireless medium

Reasons for Layer 2:

- Framing: Wi-Fi organizes bits received from the Physical layer into structured frames (802.11 frames).

- Media Access Control (MAC): Wi-Fi uses a MAC sublayer (IEEE 802.11 MAC) to manage how devices access the shared airwaves, utilizing techniques like Carrier Sense Multiple Access with Collision Avoidance (CSMA/CA).

- Node-to-Node Addressing: It utilizes MAC addresses to ensure frames are delivered to the correct device within the local network segment.

- Error Detection and Flow Control: It provides mechanisms for checking data errors and controlling data flow to ensure reliable transmission between nodes.

2. Can you share the Wi-Fi devices that you are using day to day life share that device’s wireless capability / properties after connecting to the network. Match your device to Wi-FI generation based on the capability

| Device Name | Freq Band | Max Bandwidth / Channel | Non-Overlapping Channels | Max MIMO | Highest Modulation | Max PHY Data Rate | MU-MIMO | MU-OFDMA (Shared Bandwidth) | TWT | Wi-Fi Generation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OnePlus Nord CE3 Lite 5G | 2.4 GHz, 5 GHz | 80 MHz | (approx) 6 (5 GHz) | 2×2 | 1024-QAM | (approx) 1200 Mbps | Yes | No | No | Wi-Fi 5 (802.11ac) |
| Cisco Wi-Fi 6 Router | 2.4 GHz, 5 GHz | 160 MHz | (approx) 8+ | 4×4 | 1024-QAM | (approx) 4800 Mbps | Yes | Yes | Yes | Wi-Fi 6 (802.11ax) |
| Dell Inspiron 14 (2024) | 2.4 GHz, 5 GHz | 160 MHz | (approx) 8+ | 2×2 | 1024-QAM | (approx) 2400 Mbps | Yes | Yes | Yes | Wi-Fi 6 |
| Redmi Pad 6 | 2.4 GHz, 5 GHz | 80 MHz | (approx) 6 | 2×2 | 1024-QAM | (approx) 1200 Mbps | Yes | No | No | Wi-Fi 5 (802.11ac) |
| TP-Link Home Router | 2.4 GHz, 5 GHz | 80–160 MHz | (approx) 6–8 | 2×2 or 4×4 | 1024-QAM | (approx) 1200–3000 Mbps | Yes | Partial | No/Yes | Wi-Fi 5 / Wi-Fi 6 |

3. What is BSS and ESS ?

Basic Service Set (BSS):

Basic Service Set (BSS) is a single Acess Point (AP) connecting devices within a small range (called as cell) through a wireless medium commonly used in home routers.

Extended Service Set (ESS):

Extended Service Set (ESS) connect more than one BSS usually through Ethernet and allows user within the ESS to rome freely under a single Service Set Identifier (SSID).

| Criteria | BSS | ESS |
| --- | --- | --- |
| Structure | Consist of one AP and Client | Consist of 2 or more BSSs (multiple APs) |
| Coverage | Small (one cell) | Large (multiple cells) |
| SSID | Identified by MAC address of AP (BSSID) | All BSSs in ESS share the same service set identifier (SSID) and are distinguised by BSSIDs of individual AP |
| Roaming | Not possible without disconnection | Allows user to move between APs without disconnection |
| Example | A home router | University Campus with many APs |

4. What are the basic functions of Wi-Fi access points ?

Basic Functions of Wi-Fi Access Points:

- Bridging Wired to Wireless: Connects directly to a wired Ethernet router, switch, or hub and converts that data into a wireless signal, allowing wireless devices to access the wired network.

- Extending Coverage (Repeater Mode): Acts as a repeater by extending the Wi-Fi signal to areas with low signal strength or dead spots, such as in large offices or homes.

- Wireless Connectivity Hub: Acts as the central point for wireless communications, facilitating communication between connected clients.

- VLAN Support (Segmentation): Supports multiple SSIDs, enabling segmentation of the network to provide separate connections for guests, IoT devices, or employees.

- Bridging Different Network Types: Can be configured to connect wireless-only devices to wired infrastructure, acting as a workgroup bridge.

5. Difference between Bridge Mode and Repeater Mode.\

| Bridge Mode | Repeater Mode |
| --- | --- |
| Connect two different routers | Extend the network coverage to dead xones |
| Maitain faster Speed | Reduce througput as they act as Secondary APs |
| Best for connecting Distant Wireless Devices that need stable connection without running cable | Best for boosting signals into dead zones for mobile devices |
| Entire bandwidth is used for Secondary AP | Bandwidth is not fully used for Secondary AP they are split to use across both Primary and secondary AP equally using the same Wireless channel |

6. What are the difference between IEEE 802.11a and IEEE 802.11b

| Feature | IEEE 802.11a | IEEE 802.11b |
| --- | --- | --- |
| Frequency | 5 GHz | 2.4 GHz |
| Max Speed | 54 Mbps | 11 Mbps |
| Range | Shorter | Longer |
| Interference | Low | High |
| Modulation | OFDM | DSSS |
| Channels | More | Limited (3) |
| Cost (initially) | Higher | Lower |

7. Configure your modem/hotspot to operate only in 2.4 Ghz and connect your laptop / WI-Fi device and capture the propertoes in your wifi device. Repeat the same in 5Ghz and tabulate the difference you observe during this.

Properties in 2.4Ghz

Device used for hotspot: Oneplus Nord CE3 Lite 5G (Intel(R) Wi-Fi 6E AX211 160MHz)

Device used for Connection : Dell Inspiron 14 (Wi-Fi card - Intel(R) Wi-Fi 6E AX211 160MHz)

Properties in 5 GHz:

Device used for hotspot: Oneplus Nord CE3 Lite 5G (Intel(R) Wi-Fi 6E AX211 160MHz)

Device used for Connection : Dell Inspiron 14 (Wi-Fi card - Intel(R) Wi-Fi 6E AX211 160MHz)

| Feature | 2.4 GHz | 5 GHz |
| --- | --- | --- |
| Wi-Fi Standard | Wi-Fi 4 (802.11n) | Wi-Fi 5 (802.11ac) |
| Frequency Band | 2.4 GHz | 5 GHz |
| Channel | 6 | 149 |
| Speed (Rx/Tx) | 72 / 72 Mbps | 433 / 433 Mbps |
| Range | Longer | Shorter |
| Wall Penetration | Better | Weaker |
| Interference | High (inference from Bluetooth, microwave, etc.) | Low |
| Network Stability | More stable at distance | Best when near router |

8. What is the difference between IEEE and WFA ?

| Basis of Comparison | IEEE | WFA (Wi-Fi Alliance) |
| --- | --- | --- |
| Full Form | Institute of Electrical and Electronics Engineers | Wi-Fi Alliance |
| Nature | Professional standards organization | Industry consortium |
| Primary Role | Develops technical standards | Certifies Wi-Fi devices |
| Main Function | Defines protocols (e.g., 802.11 series) | Ensures interoperability between devices |
| Focus Area | Design and specification of wireless communication | Testing, compatibility, and user experience |
| Output | Standards documents (802.11a/b/g/n/ac/ax) | “Wi-Fi CERTIFIED” label |
| Target Users | Engineers, researchers, developers | Manufacturers and consumers |
| Example | Defines how Wi-Fi works (e.g., modulation, frequency) | Tests if router and mobile can work together |
| Responsibility | Standard creation | Product validation |
| Dependency | Independent body | Uses IEEE standards for certification |

9. List down the types of Wi-Fi internet connectivity backhauls share your home/college’s wireless internet connectivity backhaul name and its properties.

Types of Wi-Fi connectivity Backhauls:

- Fibre Optics

- MoCA

- Satellite

- Cellular

- Powerline Backhaul

The type of backhaul used in our college if Fibre Optics Backhaul:

Properties of Fibre Optics Backhaul:

- Low Latency

- Immune to interference

- Capable of transfering High bandwidth over long distance

- High reliablity and security

The type of backhaul used inour home is Cellular Backhaul:

Properties of Cellular Backhaul:

- Capable of handling large volume of data from multiple user (through 4G/LTE and 5g technology)

- Rapid deployment and easy mobility

- Variable Latency unlike Fibre Optics

10. List down the Wi-Fi topology and their use cases.

Wi-Fi topologies and their use cases:

1. Infrastructure Mode

Description: Devices connect through a central Access Point (AP) (router).

Use cases:

- Home Wi-Fi networks

- College/office networks

- Internet access for multiple devices

2. Repeater Mode

Description: Extends existing Wi-Fi by receiving and re-transmitting signals.

Use cases:

- Eliminating dead zones in large homes

- Extending campus/building coverage

- Improving weak signal areas

3. Bridge Mode

Description: Connects two networks (wired or wireless) at Layer 2.

Use cases:

- Connecting two buildings wirelessly

- Linking LANs without running cables

- Backbone connections in campuses

4. Ad-Hoc Mode

Description: Direct device-to-device connection without an AP.

Use cases:

- Quick file sharing between laptops

- Temporary networks (meetings, labs)

- Peer-to-peer communication

5. Mobile Hotspot Mode

Description: Smartphone or device acts as a Wi-Fi router.

Use cases:

- Sharing mobile data

- Internet access during travel

- Backup connection when broadband fails

6. Mesh Mode

Description: Multiple nodes form a self-healing network with seamless roaming.

Use cases:

- Smart homes with large coverage

- Hotels, malls, campuses

- IoT-heavy environments

7. Workgroup Bridge Mode (WGB)

Description: Connects wired devices to a wireless network via a bridge.

Use cases:

- Connecting printers, CCTV, or legacy devices

- Industrial equipment networking

- Smart classrooms/labs

8. IoT Gateway Mode

Description: Acts as a bridge between IoT devices and cloud/network.

Use cases:

- Smart home automation

- Industrial IoT (IIoT)

- Sensor data collection and processing